BCSE417L – MACHINE VISION LAB

EXPERIMENT 10: Wavelet Transform for Image Feature Extraction and Compression

SUBMITTED BY-

22BAI1118 – Mayank Raj

SUBMITTED TO-

Trilok Nath Pandey

Date: 28/03/2025

Aim

- Perform feature extraction using Wavelet Transform on medical MRI images

- Implement image compression techniques using Discrete Wavelet Transform (DWT)

- Compare Discrete Wavelet Transform (DWT) with Fast Fourier Transform (FFT) for medical image processing

Methodology

Task 1: Feature Extraction using Wavelet Transform

Approach:

- Load MRI image in grayscale

- Apply 2D Discrete Wavelet Transform (DWT) using Haar wavelet

- Decompose image to 2 levels

- Extract and analyze:

  - Approximation coefficients (LL)

  - Horizontal (LH), Vertical (HL), and Diagonal (HH) details

Theoretical Justification:

- Haar wavelet is computationally efficient

- Provides good edge and texture detection in medical images

- Offers multi-resolution analysis

Task 2: Image Compression using DWT

Compression Technique:

- Hard thresholding of wavelet coefficients

- Zero out coefficients below 10% of maximum coefficient value

- Reconstruct image using Inverse DWT (IDWT)

Compression Metrics:

Compression Ratio: 1.00

Mean Squared Error: 9.60

Peak Signal-to-Noise Ratio: 38.31 dB

Compression Impact Analysis:

Wavelet Transform preserves key image features

Thresholding removes less significant coefficients

Trade-off between compression and image quality

Task 3: Comparison of FFT vs DWT

Comparative Analysis Methods:

- Apply Fast Fourier Transform (FFT)

- Perform frequency-domain filtering

- Compare techniques across multiple dimensions

Comparative Metrics:

- DWT Compression:

  - Compression Ratio: 1.00

  - Mean Squared Error: 9.60

  - Peak Signal-to-Noise Ratio: 38.31 dB

  - Computation Time: 0.0220 seconds

- FFT Compression:

  - Compression Ratio: 1.00

  - Mean Squared Error: 0.50

  - Peak Signal-to-Noise Ratio: 51.14 dB

  - Computation Time: 0.0665 seconds

Analysis and Insights

Feature Retention

- DWT preserves spatial and frequency information

- Maintains spatial localization better than FFT

- Haar wavelet effectively captures edge and texture details

Compression Efficiency

- Both transforms achieved a Compression Ratio of 1.00

- FFT showed higher Peak Signal-to-Noise Ratio (51.14 dB vs 38.31 dB)

- DWT provides more diagnostic-friendly image representation

Computational Performance

- DWT faster (0.0220 seconds) compared to FFT (0.0665 seconds)

- Lower computational complexity for DWT

Conclusion

- Discrete Wavelet Transform (DWT) with Haar wavelet is superior for medical image processing

- Reasons:

  - Better spatial information preservation

  - Lower computational complexity

  - Suitable for telemedicine and medical image transmission

  - Maintains diagnostic image quality